Great — let’s go through your configuration and outputs step by step and explain exactly what is happening in your IPv6 setup.

# ✅ 1️⃣ Successful IPv6 Ping

Router1# ping ipv6 2001:AAAA:BBBB:CCCC:1234:1234:1234:1234

!!!!!

Success rate is 100 percent

### What this tells us:

- Router1 successfully reached the IPv6 host:

- 2001:AAAA:BBBB:CCCC:1234:1234:1234:1234

- 100% success = Layer 1, 2, and 3 connectivity is working

- IPv6 routing and Neighbor Discovery are functioning correctly

- The device is on the same subnet as Fa0/0

# ✅ 2️⃣ FastEthernet0/0 IPv6 Configuration

FastEthernet0/0 is up, line protocol is up

This confirms:

- Physical interface is UP

- Data link layer is UP

- Interface is operational

## 🔹 Link-Local Address

FE80::260:47FF:FED7:C801

- Automatically generated

- Used for:

  - Neighbor Discovery (NDP)

  - Router Advertisements

  - DHCPv6 communication

- Only works inside the local link

Every IPv6 interface MUST have a link-local address.

## 🔹 Global Unicast Addresses

2001:AAAA:BBBB:CCCC::/64

2001:AAAA:BBBB:CCCC:260:47FF:FED7:C801/64 [EUI]

You actually have two global addresses:

### 1️⃣ Subnet-router anycast address:

2001:AAAA:BBBB:CCCC::

- This is automatically created

- It represents the subnet itself

### 2️⃣ EUI-64 Address:

2001:AAAA:BBBB:CCCC:260:47FF:FED7:C801

- Automatically generated using MAC address

- [EUI] indicates EUI-64 format

- MAC address was split and inserted into IPv6

## 🔹 Router Advertisements

ND router advertisements are sent every 200 seconds

Hosts use stateless autoconfig for addresses.

This means:

- Router1 is acting as an IPv6 router

- It is sending Router Advertisements (RA)

- Hosts can automatically generate their own IPv6 addresses using SLAAC

# ✅ 3️⃣ Manual Link-Local on Fa0/1

You configured:

ipv6 address FE80::AAAA:BBBB:CCCC:DDDD link-local

Now Fa0/1 has:

FE80::AAAA:BBBB:CCCC:DDDD

Important:

- You manually overrode the automatically generated link-local

- This is allowed

- Common in labs for control

# ✅ 4️⃣ DHCPv6 Client on Fa0/1

ipv6 address dhcp

Then:

show ipv6 dhcp interface

State is SOLICIT(5)

### What SOLICIT means:

- The router is acting as a DHCPv6 client

- It is sending DHCPv6 Solicit messages

- It is searching for a DHCPv6 server

- No server has responded yet

That is why Fa0/1 only shows:

FE80::AAAA:BBBB:CCCC:DDDD

There is no global IPv6 address assigned yet.

# ✅ 5️⃣ IPv6 Neighbors

show ipv6 neighbors

Output:

2001:AAAA:BBBB:CCCC:1234:1234:1234:1234    REACH Fa0/0

FE80::2D0:58FF:FEB6:DC63                   REACH Fa0/1

This is the IPv6 equivalent of ARP table.

IPv6 uses:
➡ NDP (Neighbor Discovery Protocol) instead of ARP.

### First Entry

2001:AAAA:BBBB:CCCC:1234:1234:1234:1234

- This is the host you pinged

- MAC address: 000C.CF6A.835B

- State: REACH (communication confirmed)

- Interface: Fa0/0

### Second Entry

FE80::2D0:58FF:FEB6:DC63

- A neighbor on Fa0/1

- Likely a router or another host

- Only link-local address

- Learned through NDP

# ✅ 6️⃣ IPv6 Routing Table

show ipv6 route

You have 4 entries.

## 🔹 Connected Route

C   2001:AAAA:BBBB:CCCC::/64

via ::, FastEthernet0/0

Meaning:

- This network is directly connected

- Router does not need next-hop

- Traffic goes directly out Fa0/0

## 🔹 Local Routes

L   2001:AAAA:BBBB:CCCC::/128

L   2001:AAAA:BBBB:CCCC:260:47FF:FED7:C801/128

These represent:

- The router's own addresses

- /128 = single host route

- Used so router knows packets destined for itself

## 🔹 Multicast Route

L   FF00::/8 via Null0

- IPv6 multicast range

- All multicast traffic

- FF00::/8 is reserved multicast block

# ❗ Important Observation

You do NOT have:

- A default route (::/0)

- Any learned dynamic routes

- Any DHCPv6-assigned global address on Fa0/1

# 🔍 Why DHCPv6 Isn’t Working

Fa0/1 shows:

State is SOLICIT

This means:

- No DHCPv6 server exists on that segment

- Or router advertisement flags (M flag) are not set

- Or no DHCPv6 server is configured

So Fa0/1 remains with only link-local.

# 📌 Summary of Entire Configuration

| Interface | Link-Local | Global | DHCPv6 | Status |
| --- | --- | --- | --- | --- |
| Fa0/0 | Auto | Yes (EUI-64) | No | Fully working |
| Fa0/1 | Manual | No | Trying | No DHCP server |

# 🔥 What Is Working

✔ IPv6 enabled
✔ Global addressing on Fa0/0
✔ SLAAC functioning
✔ NDP working
✔ IPv6 routing table correct
✔ Ping successful

# 🚫 What Is Not Working

❌ DHCPv6 client cannot find server
❌ No global IPv6 on Fa0/1
❌ No default route

# 🧠 Big Picture

Your router is:

- Acting as IPv6 router on Fa0/0

- Acting as DHCPv6 client on Fa0/1

- Successfully communicating with IPv6 neighbor

- Using NDP instead of ARP

- Using EUI-64 for global address generation

If you want, I can now explain:

- Difference between SLAAC and DHCPv6

- Why two global addresses appear

- How to configure a DHCPv6 server

- How to add a default IPv6 route

- How IPv6 Neighbor Discovery actually works internally

Just tell me which topic you want next.

Excellent — now let’s fully analyze Router2 and connect it logically to what we saw on Router1.

We’ll break this into sections so everything is crystal clear.

# ✅ 1️⃣ show ipv6 dhcp

This device's DHCPv6 unique identifier (DUID): 00030001000CCF6A835B

### What is DUID?

DUID = DHCP Unique Identifier

- Every DHCPv6 client/server has one

- It uniquely identifies the device

- Used instead of MAC address (like IPv4 DHCP)

### Breaking it down:

0003 0001 000CCF6A835B

- 0003 → DUID type (DUID-LL = based on link-layer address)

- 0001 → Hardware type (Ethernet)

- 000CCF6A835B → MAC address

So Router2’s MAC address is:

00:0C:CF:6A:83:5B

That matches what we saw earlier in neighbor tables.

# ✅ 2️⃣ show ipv6 neighbors

2001:AAAA:BBBB:CCCC::         REACH Fa0/0

FE80::AAAA:BBBB:CCCC:DDDD     REACH Fa0/1

This is the IPv6 equivalent of the ARP table.

IPv6 does NOT use ARP.
It uses Neighbor Discovery Protocol (NDP).

## 🔹 First Entry

2001:AAAA:BBBB:CCCC::

MAC: 0060.47D7.C801

Interface: Fa0/0

This is Router1.

Remember from Router1:

2001:AAAA:BBBB:CCCC::

That’s Router1’s subnet-router anycast address.

Router2 has learned:

- IPv6 address

- Corresponding MAC address

- That it is reachable

- On interface Fa0/0

State: REACH
→ Communication is active and confirmed.

## 🔹 Second Entry

FE80::AAAA:BBBB:CCCC:DDDD

MAC: 0060.47D7.C802

Interface: Fa0/1

This is Router1’s manually configured link-local on Fa0/1.

Remember you configured:

ipv6 address FE80::AAAA:BBBB:CCCC:DDDD link-local

So Router2 learned Router1’s link-local via NDP.

Important:

IPv6 routing protocols and DHCPv6 use link-local addresses for communication between routers.

# ✅ 3️⃣ show ipv6 route

IPv6 Routing Table - 3 entries

You only have 3 routes. Let’s analyze each.

## 🔹 Connected Route

C   2001:AAAA:BBBB:CCCC::/64

via ::, FastEthernet0/0

Meaning:

- This network is directly connected to Router2

- No next-hop needed

- Traffic to this subnet goes directly out Fa0/0

This is normal and correct.

## 🔹 Local Route

L   2001:AAAA:BBBB:CCCC:1234:1234:1234:1234/128

via ::, FastEthernet0/0

This is Router2’s own IPv6 address.

The /128 means:

- Single host route

- This is the router’s personal address

So Router2’s global address is:

2001:AAAA:BBBB:CCCC:1234:1234:1234:1234

This is the address Router1 successfully pinged earlier.

## 🔹 Multicast Route

L   FF00::/8 via Null0

This represents:

- All IPv6 multicast addresses

- Required for:

  - Neighbor Discovery

  - Router Advertisements

  - DHCPv6

  - OSPFv3

  - EIGRPv6

Every IPv6 device has this.

# 🚨 What Is Missing?

There is NO:

::/0   (default route)

Meaning:

Router2 cannot reach any network outside:

2001:AAAA:BBBB:CCCC::/64

It only knows about its directly connected network.

# ✅ 4️⃣ show ipv6 protocols

IPv6 Routing Protocol is "connected"

IPv6 Routing Protocol is "static"

This means:

Router2 is currently running:

- Connected routes (automatic)

- Static routing (if manually configured)

But it is NOT running:

- OSPFv3

- EIGRPv6

- RIPng

- BGP

So this is a very simple routing environment.

# 🔎 Comparing Router1 and Router2

| Feature | Router1 | Router2 |
| --- | --- | --- |
| Global IPv6 | Yes | Yes |
| DHCPv6 client | Yes (Fa0/1) | Not shown |
| Default route | No | No |
| Dynamic routing | No | No |
| Neighbor entries | Yes | Yes |
| Connected route | Yes | Yes |

# 🧠 How Communication Is Working

When Router1 pinged:

2001:AAAA:BBBB:CCCC:1234:1234:1234:1234

Here’s what happened internally:

1️⃣ Router1 checks routing table
2️⃣ Sees connected route for /64
3️⃣ Uses Neighbor Discovery to find MAC
4️⃣ Sends ICMPv6 Echo Request
5️⃣ Router2 replies
6️⃣ Neighbor state becomes REACH

That’s why ping worked perfectly.

# 📌 Important IPv6 Concepts Demonstrated Here

### 1️⃣ No ARP in IPv6

Replaced by:

- ICMPv6

- Neighbor Solicitation

- Neighbor Advertisement

### 2️⃣ Every interface has:

- Link-local address

- Global address (if configured)

### 3️⃣ IPv6 Routing Table Structure

- C → Connected

- L → Local (device’s own address)

- Static → manually added

- FF00::/8 → Multicast

# 🔥 Why Router2 Doesn’t Have DHCP Info

Router2 only shows:

show ipv6 dhcp

But it does NOT show:

show ipv6 dhcp interface

So Router2 is not acting as a DHCPv6 client.

It likely got its IPv6 via:

- Manual configuration
OR

- SLAAC from Router1

Given your earlier outputs, Router1 sends Router Advertisements:

Hosts use stateless autoconfig

So Router2 probably used SLAAC.

# 🌎 Current Network Topology (Based on Outputs)

Router1 Fa0/0  ←→  Router2 Fa0/0

2001:AAAA:BBBB:CCCC::/64

They are on the same IPv6 subnet.

Fa0/1 is another link between them using link-local communication.

# 📊 Network Behavior Summary

✔ Neighbor Discovery working
✔ SLAAC likely working
✔ IPv6 connectivity confirmed
✔ Routing tables correct
✔ No dynamic routing
✔ No default route
✔ No DHCPv6 server present

# 🎯 If You Tried to Reach Internet Now

It would FAIL because:

There is no:

::/0 default route

You would need:

ipv6 route ::/0 <next-hop>

# 💡 Big Picture

Right now this is a basic IPv6 local network lab demonstrating:

- Global unicast addressing

- EUI-64

- Link-local addressing

- Neighbor Discovery

- SLAAC

- Connected routing

- Static routing capability

If you want, next I can explain:

- Exactly how SLAAC generated Router2’s address

- How NDP works step-by-step (packet level)

- Difference between DHCPv6 and SLAAC

- How to configure full DHCPv6 server

- How to configure OSPFv3

- How to add IPv6 default routing

Tell me which direction you want to go next.